---------- Forwarded message ---------
From: Radhakrishnan Mailsamy <radhakrishnan.m@auromatrix.com>
Date: Sun, Sep 22, 2019, 20:38
Subject: Jodhpur Project Visit drawings and details-reg
To: jkjain9@gmail.com <jkjain9@gmail.com>
Cc: Kamal Chiksenia <kamalchiksenia@gmail.com



Good Evening Sir,

Regarding the Jodhpur Project Please find the drawings and below details to be followed at site.

·         All the brick walls should be replaced with Aerocon blocks.

·         The Deflection crack should be monitored with Glass strip (minimum 25mm width)

·         Remove all the existing  partially demolished brick walls .

·         All the demolished and dumped bricks over the floors to be cleared from the building  with immediate effect.

·         All the rebar drilling and fixing work  should be done with expert team only.

·         The foundation for  New columns  can be followed as per the existing drawing at site also can be laid top of the raft. The column size should                       be 9” x 30” with existing rebar details.

·         All the foundation  concrete grade should be M25.

·         The water tank columns can be followed as per the existing drawing.

·         Not suggested to cast the elevation projection slab in Concrete at the terrace level.

·         The pipe exhaust for kitchen not advisable , (Not used kitchens will be received the smoke).Air ducting system can be followed.

·         The windows elevation projections can be leave with aerocon block not in the RCC.

·         All the building area floor finishes can be ceramic or vitrified tiles not Marble/Granite.

·         The parking area Ramp existing details can be followed.